GENERAL RELEASE

GUOHE TECHNOLOGIES | 902 BAOSHENG AVENUE, YUBEL DISTRICT, CHONGQING

PMR-171

Product Datasheet v1.0 | 1st August 2023

PMR-171 Technical Datasheet

General Overview

The PMR-171 is an ultra-portable, all-band (100kHz to 2GHz) / all-mode (FT8, USB, LSB, CW, AM, FM, RTTY, DMR (optional), WFM (receive only), etc.), SDR radio, designed, manufactured, and sold by GUOHE Electronics. With its dual VFO modes, split-frequency operation support, intermediate frequency offset adjustments, fine-tuning, noise suppressions, AGC speed selections, RF gain and squelch controls, integral pre-attenuator processing, integrated AM/FM broadcast receivers, automatic sleep and transmit-time-out timers, CAT, and configuration cloning capabilities; the PMR-171 is truly a next generation field and QRP solution.

The PMR-171 leverages an externally attached modular 5AH battery-carrier designed to be an ideal complement for fully self-contained field radio deployments.  This is especially true when the modular battery carrier is augmented with dual battery backup and the radio’s native ultra-low power consumption architecture to significantly extend battery-powered operation times. To service the varied interest of modern amateur radio operators, the PMR-171 also offers a standardized external 9VDC-18VDC power input (with anti-reverse-polarity protection), allowing it to be equally at home in fixed-station and vehicle/mobile deployment scenarios.

Meant to provide an optimized operational experience in a multitude of environments, the visual interface of the PMR-171 is designed around a high-resolution, LCD display, with adjustable backlight brightness.  This, combined with the user-selectable brightness level of the keypad, allows for clear and easy readability, even under the glare of mid-day outdoor conditions.

Designed as a modern multi-function SDR platform, the PMR-171 supports the QRadioBLE mobile app (amongst others), allowing for wireless control of the radio via Bluetooth, transforming operation and control into a convenient and fast app-based experience that very effectively eliminates problems related to wired common-mode interference. When direct wired connections are desired, the integrated internal soundcard and serial control interface can still be accessed via a single USB cable connection, allowing for the support of a wide range of standard amateur radio software on a wide array of host computing platforms.

The PMR-171 has many advanced features typically available only in large base-station radios; dual VFO mode, split frequency operations, intermediate frequency offset adjustments, receive frequency fine-tuning, intermediate frequency noise suppression, AGC speed selection, RF gain adjustment, squelch control, pre-attenuator, AM/FM broadcast reception, built-in CW automatic keying, automatic key point ratio adjustment, built-in CTCSS analog subtone, automatic sleep function and transmission timeout function (TOT) timers; computer-aided control functions, and data cloning options, amongst many other features, reinforce this concept.

The PMR-171, along with its assortment of optional add-on modules (Compass, DMR, & GPS) are meant to bring joy to your QRP and Field-Radio operations.

The PMR-171 incorporates the following:

- Real-time spectrum display.

- Waterfall display.

- Doppler frequency tracking.

- Software-Defined Radio (SDR) all-band/all-mode: FT8, USB, LSB, CW, RTTY, AM, FM, DMR (optional), WFM (receive only).

- Dual frequency conversion circuit structure.

- Intermediate Frequency (IF) width and IF displacement hardware and software can be modified to provide powerful IF interference suppression.

- Digital Signal Processing (DSP), Digital Noise Reduction (DNR).

- Built-in 4m to 160m high-speed automatic antenna tuner (ATU).

- Built-in programable and electronic automatic CW keyer.

- Built-in soundcard, CAT control, and IQ logic.

- External battery compartment.

- USB TYPE-C (3.1) interface.

- High-precision TXCO ± 0.5ppm (-10°C to 60°C).

- Ultra-wide working input voltage range: 9VDC to 18VDC (some TX levels limited by input voltage).

- Power supply anti-reverse polarity protection.

- Built-in GPS/BeiDou, electronic compass, and accelerometer (acceleration, angle sensor) (optional GPS module required).

- GPS timing (optional GPS module required).

- RTC clock set capability.

- Voltage monitoring/display.

- Ultra-lightweight: ≤2kg.

- Bluetooth wireless control (viable for Bluetooth FT8.)

Targeted Applications:

- Emergency Communications,

- Remote Spectrum Monitoring/Detection,

- Radio Direction Finding,

- Amateur Radio,

PMR-171 - Datasheet

| Transmitter Specifications |  |  |
| --- | --- | --- |
| Architecture | Software Defined Radio (SDR) |  |
| Available Modes | USB, LSB, FT8, CW, RTTY, AM, FM, DMR(Matching) |  |
| Frequency Precision | 1HZ |  |
| TX Frequency range*1 (Frequency range can be customized) | ITU Region 1: 1.810-1.850, 3.500-3.800, 5.3515-5.3665, 7.000-7.200 , 10.100-10.150, 14.000-14.350, 18.068-18.168, 21-21.450, 24.890-24.990, 28-29.7, 50-54, 144-146, 430-440 (MHz) |  |
|  | ITU Region 2: 1.800-2.000, 3.500-4.000, 5.3515-5.3665, 7.000-7.300, 10.100-10.150, 14.000-14.350, 18.068-18.168, 21-21.450, 24.890-24.990, 28-29.7, 50-54, 144-148, 430-450 MHz |  |
|  | ITU Region 3: 1.800-2.000, 3.500-3.900, 5.3515-5.3665, 7.000-7.200, 10.100-10.150, 14.000-14.350, 18.068-18.168, 21-21.450, 24.890-24.990, 28-29.7, 50-54, 144-148, 430-440 MHz |  |
| Output Power | HF: SSB: (1-20W, CW: (0.1-10W, FM（0.1-20W）, AM: (1-20W） VHF: SSB/CW /FM（≤10W） UHF: SSB/CW /FM（≤10W） |  |
| Power Consumption | TX: 13.8V 5A RX: 13.8V 0.25A (nominal), 0.35A (Peak Load – full volume and screen brightness) |  |
| Carrier Suppression | ＜50db |  |
| Spurious Suppression | 1.8-54MHZ：≥50db 144-146MHZ：≥60db 430-440MHZ：≥60db |  |
| Channel Memory | 100 Channels |  |
| Receiver Specifications |  |  |
| Architecture | SDR |  |
| Available Modes | USB, LSB, FT8, CW, RTTY, AM, FM, WFM(RX-only), DMR(Matching) |  |
| RX Frequency Range | 100KHZ-2GHZ* |  |
| IF Bandwidth | 20KHZ |  |
| Intermediate Frequency | 12KHZ |  |
| Sensitivity *2 | SSB/CW: (BW: 2.4kHz @ 10dB S/N） 0.18uV (1.8-54MHZ), 0.25uV (144-148MHZ), 0.25uV (430-450MHZ), AM: (BW: 6kHz @ 10dB S/N), 15uV (0.3-1.8MHZ), 2uV (1.8-54MHZ), 2uV (144-148MHZ), 2uV (430-450MHZ), FM: (BW: 15kHz @ 12dB S/N), 0.5uV (28.0-29.7MHZ), 0.25uV (50-54MHZ), 0.3uV (144-148MHZ), 0.5uV (430-450MHZ） |  |
| IF Rejection | ≥70db |  |
| IF Suppression | ≥80db |  |
| Audio Output Power | 2 W (10% DISTORTION, 8Ω LOAD, 3KHZ) |  |
| Antenna Tuner Specifications |  |  |
| Frequency Range | 1.8-74MHZ |  |
| Impedance Range | 16.7Ω - 150Ω unbalanced (VSWR better than 1:3) |  |
| Precision | VSWR: 1:1.5 or less |  |
| Tune Speed | 2-5s (full segment @ 10s) |  |
| Activation Modes | Fully Automatic / Manual |  |
| Spectrum Specifications |  |  |
| Spectrum Width | 48K |  |
| RF Spectrum | FFT |  |
| Structural Parameters |  |  |
| Dimensions | 270mm x 166mm x 50mm (with battery) 220mm x 166mm x 50mm (without battery) |  |
| Weight | <2.4Kg (with battery & accessories) <1.7Kg (without battery or accessories) |  |
| Battery Specifications |  |  |
| Capacity | 5AH@14.2V |  |
| Operating Voltage Specifications |  |  |
| Voltage Range | 9-18V receiving range, 9-12V transmit power limit, 13.8-15V full power output, 15-18V transmit power limit. |  |
| Antenna Interface Specifications |  |  |
| Connectors | BNC (100KHz-74MHz) TNC (74MHz-520MHz) |  |
| Environmental Requirements |  |  |
| Operating Temp | -10℃ to ~60℃ |  |
| Relative Humidity | 10% - 90% |  |

Notes:

*1: The frequency range regulated according to local laws.

*2: To be calibrated, the final interpretation right belongs to the manufacturer.